Southwestern Industries, Inc.
TRAK Quik Cell

# TRAK Quik Cell Mill Specifications

- Table Size – 19 5/8” x 14”

- T-Slots (width x no. x pitch) – 5/8” x 4 x 3 1/8”

- Table Travel – 14”

- Quill Diameter – 3 15/16”

- Spindle Taper - #40 NST

- Spindle Speed Range – 250-5000 RPM

- Spindle Nose to Table – 18”

- Spindle Center to Column Face – 15 ¾”

- Maximum Head Travel – 18”

- Spindle Motor – 2HP

- Power Requirement – 220V, 3 phase, 60 Hz, 6.6 amp

- Maximum Weight on Table – 880 lbs.

- Maximum Work Capacities – Drilling 5/8”, Tapping 3/8”, Milling 2 in3/min, Boring 3 ¾”

- Net Dimensions (l x w x h), 31 ½” x 55” x 78”

- Machine Net Weight – 2750 lbs.

- Shipping Weight – 3000 lbs.

- Footprint (width x depth) – 22” x 46.5”

- Rapid Feed X, Y - 150 in/min

- Rapid Feed Z – 125 in/min

# Control Hardware Specifications

- 3-axis CNC, 3-axis DRO

- Industrial PC-based processor

- Z-axis ballscrew and motor integrated into the QCM-1 column

- Digital servo amplifiers custom designed for QMV operation

- DC servo motors rated at 280 in-oz continuous torque for the X, Y and Z axis

- Precision ground ball screws in table, saddle and ram

- Polycarbonate sealed keyboard and gasket sealed control enclosure to lock out contamination

- 9” CRT for clear presentation of prompts, status information and part graphics

- RS232 port for interface to computers

- Modular design simplifies service and maximizes uptime

- 230 V/3 phase/3.3 KVA

- Dual floppy disk drive with one drive accessible for additional part program storage

- Auxiliary Function Box for control of spray mist coolant, spindle off, a 110 AC outlet and a programmable rotary table

## Software Specifications

- AGE Profile programming allows easy part programming at the machine even with incomplete print data

- Tool compensation allows programming of the part rather than the tool path

- Two and three-axis circular interpolation makes arcs and any size hole easy to do with standard tools

- Two and three-axis linear interpolation to machine lines at any angle

- Conrad provides automatic corner radius programming with one data input

- Incremental and absolute programming can be mixed within an event

- Error messages to identify programming mistakes

- Programmable AC spindle motor control

- Override of programmed feedrate and rapid

- Spindle speed override

- Machine tool error and backlash compensation custom set for your machine

- Jog and powerfeed capabilities

- Program downloading from Off-Line CAD/CAM systems via a modified FANUC 6 post-processor

- Software programmable keys to keep procedures uncomplicated

- Part graphics displayed before machining or during run (real time)

- Math Help for finding points in a prompted format guided by graphics

- Mirror image and rotation of programmed events

- Scaling of print data

- Helical interpolation useful for thread milling

- Tool length offset including modifications to make small adjustments to cutting depth without having to change the program

- 3D graphics with selectable views

### Canned Cycles

- Posn/Drill – single point

- Bolt Hole – series of points evenly spaced around a circle

- Mill – a straight line in any direction

- Arc – any portion of a circle

- Tapping routine (for floating tap holders)

- Continue mill and arc routines

- Pocket – a rectangle, polygon, circle or irregular shape and all the material inside, includes finish cut

- Frame – a perimeter of a rectangle or circle, includes finish cut

- Repeat – mirror and rotation of programmed events

- Drill and bore routines

- Helix – for thread milling

- Copy – repeat, mirror and rotation of programmed events with the ability to edit them

# Service

- The SWI Customer Service Group provides after-sale support over the telephone via a toll-free service number - (800) 367-3165.

- Machine design makes self-service easy for when the customer is able to resolve the problem.

- Express Exchange program in which nearly all electronic and mechanical sub-assemblies are in stock.  In response to a problem, any parts you need may be shipped to arrive the next day.

# Available Options/Accessories

- Connection boxes

- Offline programming

- DNC for 3-axis CAM programs

- Remote Stop/Go switch