- Southwestern Industries, Inc.
TRAK DPMV5 Bed Milling Machine with the ProtoTRAK VM CNC

# Machine specifications

- Table size – 50” x 12”

- T-slots (number x width x pitch) – 3 x 63” x 2.52”

- Travel (X, Y, Z axis) – 40” x 20” x 19”

- Quill diameter - 4.13”

- Spindle taper - #40

- Spindle speed range RPM – 160-4000

- Spindle center to column face – 20.5”

- Spindle motor power – 5 HP

- Power requirement – 50 HZ/60 HZ; 3 phase, 220V, 40 FLA

- Machine net weight on table – 4400 lbs.

- Maximum weight of workpiece – 1320 lbs.

- Height of table from bottom of bed – 48”

- Maximum spindle nose to table – 19”

- Minimum table height – 85”

- Maximum table height – 100”

- Width of machine including table – 94”

- Length with electric box door closed – 81”

- Overall width including full table travel – 136”

- Overall length with electrical door open – 105”

- Footprint – 24” x 48.4”

- Way surface – dovetail X axis; square ways Y and Z axis

- Weight (approximate) net – 4400 lbs.

- Weight (approximate) shipping – 4700 lbs.

- Pallet size – 6’ x 6’

- Maximum work capacities – Drilling (mild steel) max. dia. 1.00”; tapping, 5/8”; milling capacities, 5 inch3/min

- Way surfaces hardened and ground

- Turcite coating on all slide ways

- Automatic lubrication pump

- Quill diameter 4.13"

- ProtoTRAK VM CNC - the control on the TRAK V Series Bed Mills

# Hardware Specifications

- Two or three-axis CNC, three-axis DRO

- Pentium processor with built-in video and Ethernet cards

- 32 MB or RAM with expansion slots available

- Hard drive

- TEAC floppy disk drive

- Ports and connectors: P/S 2 keyboard and mouse, RS232, RJ45

- 10.4” color active-matrix display

- Override of programmed feedrate and rapid with graphical indicator

- Polycarbonate sealed membrane with LED status lights

- Gasket sealed enclosures

- Brushless A.C. servomotors rated at 704 in-oz continuous torque

- Integrated motor encoders with .000025" underlying resolution.

- 250 IPM rapid traverse

- Precision ground ballscrews in the table, saddle, and column

- Programmable spindle speed

- Override of programmed spindle speed with graphical indicator

- Electronic handwheels

- Yaskawa AC spindle drive

- Digital servo amplifiers custom designed for mill operation

- Auxiliary functions: spray mist, coolant, spindle off and an input to a programmable rotary table or indexer

- Tapping with a floating tapping head

Software Specifications

- Windows operating system with access to Windows desktop

- Versatile: select between manual, two or three-axis CNC

- Auto Geometry Engine

- Two or three-axis circular interpolation

- Linear interpolation

- Six modes of operation

- Color graphics with adjustable views

- Icon-prompted tool set-up

- Automatic diameter cutter compensation

- Tool length offsets with modifiers

- Advanced diagnostic routines

- Jog and Power feed in DRO mode

- Selectable fine/course manual traverse

- Go To – traverse stops at dimension selected

- Automatic return to absolute zero

- Incremental and Absolute position readout

- Teach-in of manual moves

- Inch/mm selectable

- Scaling of print data

- Alpha-numeric program names

- Event comments

- Selectable tool path or part geometry programming

- Convenient canned cycles for all geometry

- Circular, rectangular or irregular pockets, islands and profiles

- Helix and thread milling routines

- Repeat of programmed events

- LOOK – program graphics with a single button push

- List step – graphics with programmed events displayed

- Conrad – one input for automatic corner radius

- Incremental and Absolute programming

- Subroutine and copy of programmed events

- Mirror with selectable cutting order

- Rotate

- Program diagnostics

- CAD/CAM interface

- Math helps with prompted graphical interface

- Tool stepover adjustable for pocket routines

- Selectable ramp or plunge cutter entry

- Spreadsheet editing

- Global data change

- Clipboard to copy events between programs

- Trial run

- TRAKing – manual control of CNC program run

- Real time run graphics with tool icon

- Access to networked drives

- Automatic file back-up routine

# Service

- The SWI Customer Service Group provides after-sale support over the telephone via a toll-free service number - (800) 367-3165.

- Machine design makes self-service easy for when the customer is able to resolve the problem.

- Express Exchange program in which nearly all electronic and mechanical sub-assemblies are in stock.  In response to a problem, any parts you need may be shipped to arrive the next day.

Available Options/Accessories

- TRAK Sensors for table and saddle

- Glass scales for table and saddle

- Power drawbar

- Remote stop/go switch

- Coolant pump

- Offline programming software

- Converters

- Limit switches

- Halogen work lamp