Southwestern Industries, Inc.
TRAK TRL 1440S
with ProtoTRAK SL CNC

# TRAK TRL 1440S Specifications

- Height of centers – 7”

- Distance between centers – 40”

- Swing over bed – 14”

- Swing over saddle wings – 13 3/16”

- Swing over cross slide – 7 13/16”

- Cross slide travel 8”

- Tool section max – ¾” x ¾”

- Coolant – 2.5 gallon

- Bed width – 10”

- Bed height – 10 ¼”

- Spindle nose – Camlock D1-5

- Spindle through hole – 1 5/8”

- Spindle taper – 1 29’ 15”

- Taper in reduction sleeve – MT-4

- Spindle diameter at front bearing – 2.76”

- Number of spindle speeds – 18

- Spindle speed range – 35-2400 RPM

- Quill travel – 5 ½”

- Quill diameter – 1 7/8”

- Quill taper hole - MT-3

- Main motor – 2.5/4.5 HP (230V)

- Coolant pump motor – 0.07 HP

# Control Hardware Specifications

- Two-axis CNC, two-axis DRO

- Intel Pentium processor with built-in video Ethernet cards

- 32 MB of RAM with expansion slot available

- Flash drive

- TEAC floppy drive

- Ports and connectors:  RS232, network (RJ45), P/S 2 keyboard and mouse

- 10.4” color active-matrix LCD

- 100 IPM rapid traverse

- Precision ground ball screws for cross slide and longitudinal travel

- Override of programmed feedrate and rapid with graphical indicator

- Polycarbonate sealed membrane with LED status lights

- Gasket sealed enclosures

- Electronic handwheels

- Jogstick for convenient jog

- Magnetek AC spindle drive

- Digital servo amplifiers custom designed for lathe operation

- Servo motors – 280 in-oz torque

Software Specifications

- Windows operating system

- Automatic cutter compensation

- Circular interpolation

- Linear interpolation

- Look – Program graphics with a single button push

- Six modes of operation

- Color graphics with adjustable views

- Tool offset with modifiers

- Advanced diagnostic routines

- Jog and Powerfeed in DRO mode

- Automatic return to ABS zero

- Incremental and Absolute position readout

- Inch/mm selectable

- Alpha-numeric program names

- Event comments

- Selectable Tool Path or Part Geometry programming

- Convenient canned cycles for all geometry

- Repeat of programmed events

- List step – graphics with programmed events displayed

- Chamfer/conrad to blend lines and arcs

- Incremental and Absolute programming

- Program diagnostics

- CAD/CAM interface

- Math Helps with prompted, graphical interface

- Spreadsheet editing

- Global data change

- Clipboard to copy events between programs

- Real time run graphics with tool icon

- Access to networked drives

- Automatic file back up routine

- Program converters

- Infinite threading range of standard and multi-lead threads

- Switchable handwheel feeds between coarse and fine.

- Do One routines to machine tapers, radii and fillets.

- Manual Go To.

- TRAKing®

# Service

- The SWI Customer Service Group provides after-sale support over the telephone via a toll-free service number - (800) 367-3165.

- Machine design makes self-service easy for when the customer is able to resolve the problem.

- Express Exchange program in which nearly all electronic and mechanical sub-assemblies are in stock.  In response to a problem, any parts you need may be shipped to arrive the next day.

# Available Options/Accessories

- Collet Closer

- Chuck

- Offline Programming

- Steady Rest

- Follow Rest

- Face Plate

- Tooling Kit

- Remote Stop/Go Switch

- 64 Meg flash program storage