Southwestern Industries, Inc.
TRAK 1840V  
with the ProtoTRAK VL CNC

# TRAK TRL 1840 V Machine Specifications

- Height of Centers- 9”

- Distance between centers- 40 ¼”

- Swing over bed – 18 ½”

- Swing over saddle wings – 17”

- Swing over cross slide – 9”

- Cross slide travel – 13”

- Max. Recommended Dia. in Steel – 18”

- Tool section max – 1”

- Coolant – 13 gal.

- Bed Width – 14 ½”

- Bed Height – 13 3/8”

- Spindle Nose – D1-6

- Spindle through hole – 2.36”

- Spindle Taper – MT #6

- Taper in Reduction Sleeve – MT #4

- Spindle Diameter Front Bearing – 3.35”

- Double Tapered Roller Bearings

- Number of Bearings - 3

- Bearing Class (Radial Runout) – P5

- Number of Spindle Speed Ranges - 2

- Spindle Speed Ranges (RPM) – 80-850 ; 250-2500

- Spindle H.P. - 10

- Voltage - 220

- Amps, Full Load - 36

- Phase/HZ – 3/60

- Net Inches LxWxH lbs. –91 x 53 x 70, 4500

- Ship Inches LxWxH lbs. – 97 x 58 x 72, 5170

- Coolant Pump Motor – 1/8HP

- Spindle Motor Brake – Dynamic Braking

- Way Surface Hardness – 400-450 HB

- Head Stock Lubrication – Oil Bath

- Tail stock quill travel - 6.3"

- Quill diameter - 2.95"

- Quill taper hole - MT#5

- Sliding door to control chips and coolant with one door removable

- Meehanite cast monolithic base

- Heavy ribbed base construction

- Bedways hardened and ground

- Sliding surfaces Turcite coated

- Dovetails on cross slide, V-ways on saddle

- Adjustable saddle gibs

- ProtoTRAK VL CNC - the control on the TRAK V Series Lathes

# Hardware Specifications

- Two-axis CNC, two-axis DRO

- Pentium processor with built-in video and Ethernet cards

- 32 MB or RAM with expansion slots available

- Hard Drive

- TEAC floppy disk drive

- Ports and connectors: P/S 2 keyboard and mouse, RS232, RJ45

- 10.4” color active-matrix display

- Brushless A.C. spindle motors rated at 704 in-oz continuous torque

- Integrated motor encoders with .000025” underlying resolution

- 250 IPM rapid traverse

- Precision ground ballscrews for cross slide and longitudinal travel

- Programmable spindle speed

- Polycarbonate sealed membrane with LED status lights

- Gasket sealed enclosures

- Electronic handwheels

- Jogstick for convenient jog

- Yaskawa AC spindle drive

- Digital servo amplifiers custom designed for lathe operation

## Software Specifications

- Windows operating system with access to Windows desktop

- Constant Surface Speed Programming

- Feedrate programmable in inches per Revolution or Inches per Minute

- Circular interpolation

- Linear interpolation

- Six modes of operation

- Color graphics with adjustable views

- Icon-prompted tool setup

- Automatic cutter compensation

- Tool Modifiers

- Advanced diagnostic routines

- Jog and Powerfeed in DRO mode

- Selectable fine/course manual traverse

- Go To- traverse stops at dimension selected

- Automatic return to home

- Incremental and Absolute position readout

- Inch/mm selectable

- Alpha-numeric program names

- Event comments

- Convenient canned cycles for all geometry

- Cycle event for automatic roughing passes

- Repeat of programmed events

- LOOK – program graphics with a single button push

- List step- graphics with programmed events displayed

- Conrad – one input for automatic corner radius

- Chamfer – one input for chamfer

- Incremental and Absolute programming

- Program diagnostics

- CAD/CAM interface

- Math helps with prompted graphical interface

- Spreadsheet editing

- Global data change

- Clipboard to copy events between programs

- TRAKing – manual control of CNC program run

- Real time run graphics with tool icon

- Access to networked drives

- Automatic file back-up routine

- Threading in an infinite range of standard and multi-lead threads

- Software interface with optional indexer or gang tooling

# Service

- The SWI Customer Service Group provides after-sale support over the telephone via a toll-free service number - (800) 367-3165.

- Machine design makes self-service easy for when the customer is able to resolve the problem.

- Express Exchange program in which nearly all electronic and mechanical sub-assemblies are in stock.  In response to a problem, any parts you need may be shipped to arrive the next day.

# Available Options/Accessories

- Collet Closer –5C, D Camlock

- Chuck : 8” D1-6

- Steady Rest – solid brass nibs, ¾”- 6”

- Offline Software

- Tooling Kit – ¾”

- Indexer – ¾”, 8 position

- Gang Tooling – ¾”