# Southwestern Industries, Inc.
DPM E2 Bed Mill 
with ProtoTRAK Edge CNC

# TRAK DPM E2 Specifications

Table Size: 49" x 9"

T-Slots: 3 x .63” x 2.5”

Travel (X, Y, Z): 31 x 16 x 25”

Maximum quill travel: 5”

Quill diameter: 3 3/8”

Spindle Taper: R8

Spindle Speed Range: 60 - 5000 rpm

Spindle Center to Column Face: 18”

Spindle motor power: 3 HP, Inverter Duty Motor

Power Requirement control: 110V; 1P; 8A

Power Requirement machine: 220V; 3P; 8.5A

Maximum Weight on Table: 1320 lb

Height of table from bottom of bed: 33 ½”

Maximum spindle nose to table: 27”

Minimum height: 90”

Maximum height: 98”

Width of machine including table: 70”

Length with electric box door closed: 64”

Overall width including full table traverse: 104”

Footprint of machine: 23” x 40”

Weight net/shipping lbs.: 3200/3500

Maximum work capacities in mild steel: drilling, 1” dia; tapping, 5/8”; milling, 2 in3/min

Maximum rapid feed: 100 IPM

Features and Specifications for the ProtoTRAK Edge

# Hardware

- Digital Servo Amplifiers - custom designed for ProtoTRAK operation

- D.C. Servo Motors - rated at 280 in-oz. Continuous torque are twice that required

- Precision Ground Ball Screws - in the table and saddle

- Modular Design - simplifies service and maximized uptime

- 115V/60HZ/15 amps

- Feedrate Override of programmed feedrate and rapid

- Polycarbonate Sealed Membrane Keypad and Gasket Sealed Control

- Enclosures to lock out contamination

- 7.4- LCD with adjustable contrast

- Floppy Disk Storage for part programs

- Rugged Industrial PC

Software

- Transition between Manual, 2-axis CNC and (for the Edge 3) 2 1/2-axis CNC

- Diameter Cutter Compensation - allows programming of the part rather than the center of the tool path

- Circular interpolation - makes arcs and any size hole easy to do with standard tools

- Linear Interpolation - to machine lines at any angle

- Conrad - provides automatic corner radius programming with one data input

- Incremental and Absolute - programming can even be mixed within an event

- Error Messages - to identify programming mistakes

- Fault Messages - for system self-diagnostics

- Program Downloading - from offline CAD/CAM systems via a modified FANUC 6 post-processor (not available for 2 1/2-axis CNC operation of the Edge 3)

- Teach - to program manual position and mill moves

- Parts Graphics display

- Look - a single button press to view graphics during programming

- Math Help - for finding points in a prompted format with graphical representation of prompts

- Machine Tool Error Compensation and Backlash Compensation custom set on mill after installation

- Selectable Inch/mm measurement readout

- Jog of X and Y from 1 to 100 inches per minute

- Continue Mill to eliminate repetitive data inputs while programming

Programming Canned Cycles

- Posn/Drill - single point

- Bolt Hole - series of points evenly spaced around a circle

- Mill - straight line in any direction

- Arc - any portion of a circle

- Pocket - a rectangle or circle and all the material inside, includes finish cut

- Frame - a perimeter of a rectangle or circle, includes finish cut

- Repeat - of programmed events with or without offset

# Service

- The SWI Customer Service Group provides after-sale support over the telephone via a toll-free service number - (800) 367-3165.

- Machine design makes self-service easy for the customer.

- Express Exchange program in which nearly all electronic and mechanical sub-assemblies are in stock.  In response to a problem, any parts you need may be shipped to arrive the next day.

# Available Options/Accessories for the DPM E2

- Off-line Programming for running the ProtoTRAK software on the PC

- Remote Stop/Go (RSG) switch

- Glass Scale on quill for Z-axis readout

- Coolant pump

- Limit switches

- Halogen work lamp

- Chip tray/splash shield

- Tableguard

- Power drawbar