Southwestern Industries, Inc.
TRAK K4S Knee Mill 
with the ProtoTRAK SM CNC

# TRAK K4S Machine Specifications

- Table Size –50” X 10”

- T-Slots – 5/8” x 3 x 2 ½”

- Table Travel – 32”

- Saddle Travel – 16 ½”

- Knee Travel – 16 ½”

- Ram Travel – 24”

- Maximum Quill Travel – 5”

- Quill Diameter – 3 15/16”

- Spindle Taper – NST 40

- Spindle Speed Range – 70-3950 RPM

- Head Tilt –90 deg. left, 90 deg. right

- Spindle Motor – 5 HP

- Power Requirement Control – 110V; 1P; 10A

- Power Requirement Machine – 220/440V; 3P; 14/7A

- Maximum Weight on Table – 850 lbs.

- Machine Weight –3630 lbs.

- Machine dims l, w, h - 71” x 59” x 9”

- Maximum rapid feed – 100 IPM

- Way surface type – Dovetail X, Square Y, Z

- Precision 7207 CP4 spindle bearings

- Chrome hardened and ground quill

- Meehanite castings

- Slide ways are Turcite coated

- Wide way surfaces are hardened and ground

- ProtoTRAK SM CNC - the control on the TRAK S Series Knee Mill

# Hardware Specifications

- Two or three-axis CNC, three-axis DRO

- Pentium processor with built-in video and Ethernet cards

- 32 MB or RAM with expansion slots available

- Flash drive

- TEAC floppy disk drive

- Ports and connectors: P/S 2 keyboard and mouse, RS232, RJ45

- 10.4” color active-matrix display

- Override of programmed feedrate and rapid with graphical indicator

- Polycarbonate sealed membrane with LED status lights

- Gasket sealed enclosures

- Servo motors – 280 in-oz torque

- Precision ground ballscrews installed in the table and saddle

- Digital Servo Amplifiers custom tuned for knee mill operation

- Software Specifications

- Windows operating system

- Auto Geometry Engine

- Automatic diameter cutter compensation

- Circular interpolation

- Rotate

- LOOK – Program graphics with a single button push

- Linear interpolation

- Six modes of operation

- Color graphics with adjustable views

- Tool length offset with modifiers

- Advanced diagnostic routines

- Jog and Powerfeed in DRO mode

- Automatic return to ABS zero

- Incremental and Absolute position readout

- Inch/mm selectable

- Teach-in of manual moves.

- Alpha-numeric program names

- Scaling of print data

- Event comments

- Selectable Tool Path or Part Geometry programming

- Convenient canned cycles for all geometry

- Circular, rectangular or irregular pockets, islands and profiles

- Subroutine and copy of programmed events

- Mirror with selectable cutting order

- List step – graphics with programmed events displayed

- Conrad – one input for automatic corner radius

- Incremental and Absolute programming

- Program diagnostics

- CAD/CAM interface

- Math Helps with prompted, graphical interface

- Tool stepover adjustable for pocket routines

- Selectable ramp or plunge cutter entry

- Spreadsheet editing

- Global data change

- Clipboard to copy events between programs

- Trial run

- Real time run graphics with tool icon

- Access to networked drives

- Automatic file back up routine

- Program converters

# Service

- The SWI Customer Service Group provides after-sale support over the telephone via a toll-free service number - (800) 367-3165.

- Machine design makes self-service easy for when the customer is able to resolve the problem.

- Express Exchange program in which nearly all electronic and mechanical sub-assemblies are in stock.  In response to a problem, any parts you need may be shipped to arrive the next day.

- Available Options/Accessories

- Three-Axis CNC

- TRAK Sensors

- Glass Scales for table and saddle (standard on quill)

- Remote Stop/Go Switch

- Offline Software

- Limit Switches

- Converters

- Power Drawbar

- Halogen Worklamp

- Chip Pan

- Riser Block

- Knee Power Feed

- Flash Memory

- Coolant Pump