Southwestern Industries, Inc.
TRAK K4SX-3 Specifications

with the ProtoTRAK SX3 Control

# Machine Specifications

- Table Size –50” X 10”

- T-Slots – 5/8” x 3 x 2 ½”

- Table Travel – 32”

- Saddle Travel – 16 ½”

- Knee Travel – 16 ½”

- Ram Travel – 19”

- Maximum Quill Travel – 5”

- Quill Diameter – 3 15/16”

- Spindle Taper – NMTB 40

- Spindle Speed Range – 70-3950 RPM

- Head Tilt –90 deg. left, 90 deg. right

- Spindle Motor Power, Vari-speed head  - 5HP

- Spindle Motor Power, Optional E head - 5HP

- Power Requirement Control – 110V; 1P; 10A

- Power requirements, machine with vari-speed head  - 220/440V;3P; 14/7A

- Power requirements, machine with Optional E head - 220V;3P; 17.5A

- Maximum Weight on Table – 850 lbs.

- Machine Weight –3630 lbs.

- Machine dims l, w, h - 71” x 59” x 9”

- Maximum rapid feed – 100 IPM

- Way surface type – Dovetail X, Square Y, Z

- Precision 7207 CP4 spindle bearings

- Chrome hardened and ground quill

- Meehanite castings

- Slide ways are Turcite coated

- Wide way surfaces are hardened and ground

# Machine Options

- Programmable Electronic Head

- TRAK Sensors on table and saddle

- Glass Scales on table and saddle

- Auxiliary Function hardware box

- Programmable Electronic Head

- Electronic Handwheels

- Remote Stop/Go switch

- Power Drawbar

- Halogen Worklamp

- Chip Pan

- Riser Block

- Knee Power Feed

- Coolant Pump

- Auto Lube Pump

- Spray Coolant

- Table Guard Enclosure

- Limit Switches

- Vise

##### ProtoTRAK SMX System Specifications

(O) indicates optional feature

### ProtoTRAK System Hardware

- ProtoTRAK SMX CNC

- Two- or three-axis CNC, 3-axis DRO

- Real handwheels for manual operation

- 10.4” color active-matrix screen

- Industrial-grade Celeron® processor

- 256 MB Ram

- P/S 2 Keyboard connector

- 2 USB connectors

- Override of program feedrate

- LED status lights built into display

- TEAC floppy drive

- RJ45 Port and Ethernet card (O)

- Override of program spindle speed (O)

- 128 MB USB Thumb Drive flash memory (O)

- Uncluttered front panel with few hard keys

- Ballscrew and motor assembly installed on quill

- Electronic quill handwheel

### Software Features – General Operation

- Clear, uncluttered screen display

- Prompted data inputs

- English language – no codes

- Soft keys - change within context

- Windows® operating system

- Selectable two or three-axis CNC

- Color graphics with adjustable views

- Inch/mm selectable

- Convenient modes of operation

### DRO Mode features

- Incremental and absolute dimensions

- Jog at rapid with override

- Powerfeed X, Y or Z

- Do One CNC canned cycle

- Teach-in of manual moves

- Servo return to 0 absolute

- Tool offsets from library

- Go To Dimensions (O)

- Spindle speed setting with manual override (O)

- Fine/Course handwheel resolution (O)

### Program Mode features

- Auto Geometry Engine (O)

- Geometry-based programming

- 3-axis geometry programming (O)

- Tool Path programming (O)

- Scaling of print data (O)

- Multiple fixture offsets (O)

- Programming of Auxiliary Functions (O)

- Event Comments (O)

- Three-axis Geometry conversational programming (O)

- Incremental and absolute dimensions

- Automatic diameter cutter comp

- Circular interpolation

- Linear interpolation

- Look –graphics with a single button push

- List step – graphics with programmed events displayed

- Alphanumeric program names

- Program data editing

- Program pause

- Conrad – automatic corner radius

- Programmable spindle speeds (O)

- Math helps with graphical interface

- Auto load of math solutions

- Tool step over adjustable for pocket routines

- Pocket bottom finish pass

- Selectable ramp or plunge cutter entry

- Subroutine repeat of programmed events

- Nesting

- Rotate about Z axis for skewing data

- Mirror of programmed events (O)

- Copy (O)

- Copy rotate (O)

- Copy mirror (O)

### Canned cycles

- Position

- Drill

- Bolt Hole

- Mill

- Arc

- Circle pocket

- Rectangular pocket

- Irregular Pocket (O)

- Circular profile

- Rectangular profile

- Irregular Profile(O)

- Circle Island (O)

- Rectangular Island (O)

- Irregular Island(O)

- Helix (O)

- Thread milling (O)

- Engrave(O)

- Tapping(O)

### Edit mode Features

- Delete events

- Erase program

- Spreadsheet editing (O)

- Global data change (O)

- G-Code editor (O)

- Clipboard to copy events between programs (O)

### Set Up Mode Features

- Program diagnostics

- Advanced tool library

- Tool names

- Tool length offset with modifiers

- Advanced diagnostic routines

- Software travel limits

- Tool path graphics with adjustable views

- Program run time estimation clock (O)

### Run Mode Features

- TRAKing (O)

- Trial run at rapid

- 3D CAM file program run

- 3D G code file run with tool comp

- Real time run graphics with tool icon

- Countdown clock to next pause or tool change (O)

### Program In/Out Mode Features

- Simple program storage to floppy

- CAM program converter

- Converter for prior-generation ProtoTRAK programs

- DXF/DWG file converter (O)

- Selection of file storage locations (O)

- Automatic file back-up routine (O)

- Preview graphics for unopened files (O)

- Networking (O)

Control Options

### Advanced Features Option

- Auto Geometry Engine ™

- Programmable Auxiliary functions

- 3-axis conversational programming

- Additional Canned Cycles:

- Irregular Pocket

- Circle Island

- Rectangular Island

- Irregular Island

- Irregular Profile

- Helix

- Thread milling

- Engrave

- Tapping

- G-Code editor

- Countdown clock to next pause or tool change

- Total program time estimator

- Spreadsheet editing

- Global data change

- Scaling of print data

- Multiple fixture offsets

- Event comments

- Tool path conversational programming

- Mirror of programmed events

- Copy with or without offsets

- Copy Rotate

- Copy Mirror

- Clipboard to copy events between programs

### Networking/Memory Option

- Directory/File/Folder Program organization

- Automatic file back up routine

- Preview Graphics for unopened files

- USB Thumb Drive flash memory, 128 MB or more

- Networking via RJ 45 port

### The DXF File Converter Option

Import and convert CAD data into ProtoTRAK programs

DXF or DWG files

Chaining

Automatic Gap Closing

Layer control

Easy, prompted process you can do right at the machine

### CAM Out Converter Option

Save ProtoTRAK files as CAM files for running on different controls

### TRAKing/Electronic Handwheels Option

Electronic Handwheels on X and Y (replaces the mechanical handwheels)

TRAKing of programs during program run

Go To Dimensions

Selectable Fine/Coarse handwheel resolution