Southwestern Industries, Inc.
TRL 1630HSSX Lathe Specifications

with the ProtoTRAK SLX Control

# Machine Specifications

- Height of Centers-8”

- Distance between centers-30”

- Swing over bed -16”

- Swing over saddle wings-16”

- Swing over cross slide-8.6”

- Cross slide travel-8.5”

- Tool section max-3/4”

- Coolant tank capacity – 9.3 gal.

- Width of Bed-12.6”

- Height of Bed-12.6”

- Spindle Nose - A2-4

- Spindle through hole-1.57”

- Spindle taper-MT#5

- Taper in reduction sleeve -N/A

- Spindle dia at front bearing-2.56”

- Number of bearings-5

- Spindle speed range (RPM)-150-4000

- Optional Tailstock Quill travel-5”

-	Quill diameter-2.36”

-	Quill taper hole-MT#4

- Spindle Motor HP -7.5

- Power requirements, control-110V; 1P; 8 A

- Power requirements, machine-220V;3P;60Hz

- Amps full load current-25

- Dimensions net - 79 x 40 x 56”

- Dimensions shipping - 87 x 45 x 67”

- Weight net - 2,875 lbs.

- Weight shipping – 3,420 lbs.

- Maximum rapid feed X - 100

- Maximum rapid feed Z - 150

- Coolant pump motor -1/8 HP

- Spindle motor brake - Dynamic braking

- Way surface hardness - 400 – 450 HB

- Head stock lubrication - Grease

- Sliding door for easy set-ups

- Bedways hardened and ground

- Sliding surfaces Turcite coated

- Dovetails on cross slide, V-ways on saddle

- Adjustable saddle gibs

Machine Options

- Collet closer

- Chuck

- Coolant pump

- Indexer – 4 or 8-station

- Work lamp

- Remote stop/go switch

- Steady rest

- Tooling kit

- Tailstock

##### ProtoTRAK SLX System Specifications

(O) indicates optional feature

### ProtoTRAK System Hardware

- ProtoTRAK SLX CNC

- Two-axis CNC, two-axis DRO

- Electronic handwheels for manual operation

- 10.4” color active-matrix screen

- Industrial-grade Pentium® processor

- 1 GB Ram

- 4 USB connectors

- RJ45 port and Ethernet card (O)

- Override of program feedrate

- Override of program spindle speed

- LED status lights built into display

- Gasket-sealed enclosures

- Jogstick for convenient jog

- USB Thumb drive flash memory 512 MB or more (O)

- Clean front panel with few hard keys

### Software Features – General Operation

- Clear, uncluttered screen display

- Prompted data inputs

- English language – no codes

- Soft keys - change within context

- Windows® operating system

- Color graphics with adjustable views

- Inch/mm selectable

- Convenient modes of operation (see below)

### DRO Mode features

- Incremental and absolute dimensions

- Jog at rapid with override

- Powerfeed X or Z

- Tapers of any angle

- Radius

- Fillet

- Go To dimensions (O)

- Servo motor return to Home

- Spindle speed setting with override

- Tool offsets from library

- Fine/course handwheel resolution (O)

### Program Mode features

- Geometry-based programming

- Event comments (O)

- Incremental and absolute dimensions

- Automatic tool nose radius compensation

- Circular interpolation

- Linear interpolation

- Look –graphics with a single button push

- List step – graphics with programmed events displayed

- Alphanumeric program names

- Conrad – one input for automatic corner radius programming

- Chamfer – one input for automatic chamfer programming

- Math helps with graphical interface

- Auto load of math solutions

- Subroutine repeat of programmed events

- Nesting

- Programmable spindle speeds

- Program run time clock in Run Mode (O)

- CSS (Constant Surface Speed) programming  (O)

- IPR (Inch per Revolution) programming (O)

- Gang tooling  (O)

### Canned cycles

- Position

- Drill

- Bore

- Turn

- Arc

- Cycle

- Thread

- Groove

- Custom thread (O)

- Tap (O)

### Edit mode Features

- Delete events

- Erase program

- Spreadsheet editing (O)

- Global data change (O)

- Clipboard to copy events between programs (O)

### Set Up Mode Features

- Program diagnostics

- Advanced tool library

- Tool library file save

- Icon-prompted tool setting

- Tool offsets with modifiers

- Single tool set-up

- Gang tool set-up (O)

- Indexer tool set-up (O)

- Advanced diagnostic routines

- Software travel limits

- Tool path graphics with adjustable views

- Program run time estimation clock

### Run Mode Features

- CAM file program run

- Real time run graphics with tool icon

- Countdown clock to next pause or tool change (O)

- TRAKing of programs during program run (O)

- Automatic indexing for tool change (O)

- Gang tool operation (O)

### Program In/Out Mode Features

- Program storage to USB flash drive

- CAM program converter

- Converter for prior-generation ProtoTRAK programs

- DXF/DWG file converter (O)

- Selection of file storage locations (O)

- Automatic file back-up routine (O)

- Preview graphics for unopened files (O)

- Networking (O)

Control Options

### Advanced Features with Verify Option

- Verify – see a 3-D model machined before cutting chips

- Spreadsheet editing

- Global data change

- Clipboard to copy events to another program

- Event comments

- Program run time estimator

- CSS (Constant Surface Speed) programming

- IPR (Inch per Revolution) programming

- Gang tooling

- Additional Canned Cycles:

- Custom thread

- Tap

### Networking Option

- Networking via RJ 45 port

### The DXF File Converter Option

- Import and convert CAD data into ProtoTRAK programs

- DXF or DWG files

- Chaining

- Automatic Gap Closing

- Layer control

- Drawing Line Edit

- Easy, prompted process you can do right at the machine

### CAM Out Converter Option

- Save ProtoTRAK files as CAM files for running on different controls

### TRAKing/Electronic Handwheels Option

- TRAKing of programs during program run

- Go To Dimensions

- Selectable Fine/Coarse handwheel resolution