Southwestern Industries, Inc.
TRL 1840SX Lathe Specifications

with the ProtoTRAK SLX Control

# Machine Specifications

- Height of Centers - 9”

- Distance between centers - 40 1/4"

- Swing over bed -18  1 / 2”

- Swing over saddle wings -17”

- Swing over cross Slide - 9”

- Cross slide travel - 13”

- Tool section max - 1”

- Coolant tank capacity - 13 gal.

- Width of Bed - 14  1/ 2”

- Height of Bed - 13  3 /8”

- Spindle Nose (Camlock) - D1-6

- Spindle through hole - 2.36”

- Spindle taper - MT#6

- Taper in reduction sleeve - MT#4

- Spindle dia at front bearing - 3.35”

- Number of bearings - 3

- Bearing class (radial round)- P5

- Spindle speed range (RPM)- 80-850, 250-2500

- Tailstock Quill travel - 7 1 /8”

-	Quill diameter - 2.96”

-	Quill taper hole - MT#5

- Spindle Motor HP - 10

- Power requirements, control - 110V; 1P; 8 A

- Power requirements, machine - 220V;3P;60Hz

- Amps full load current - 33

- Dimensions net - 91 x 53 x 70”

- Dimensions shipping - 97 x 58 x 72”

- Weight net -  4500

- Weight shipping - 5170

- Maximum rapid feed X - 100

- Maximum rapid feed Z - 150

- Coolant pump motor - 1/8 HP

- Spindle motor brake - Dynamic braking

- Way surface hardness - 400 – 450 HB

- Head stock lubrication - Oil bath

- Meehanite cast monolithic base

- Heavy ribbed base construction

- Bedways hardened and ground

- Sliding surfaces Turcite coated

- Dovetails on cross slide, V-ways on saddle

- Adjustable saddle gibs

# Machine Options

- Collet closer

- Chuck

- Coolant pump

- Face plate

- Follow rest

- Gang tooling

- Indexer – 4 or 8-station

- Work lamp

- Remote stop/go switch

- Steady rest

- Tooling kit

##### ProtoTRAK SLX System Specifications

(O) indicates optional feature

### ProtoTRAK System Hardware

- ProtoTRAK SLX CNC

- Two-axis CNC, two-axis DRO

- Electronic handwheels for manual operation

- 10.4” color active-matrix screen

- Industrial-grade Celeron® processor

- 256 Mb Ram

- P/S 2 Keyboard connector

- 2 USB connectors

- RJ45 port and Ethernet card (O)

- Override of program feedrate

- Override of program spindle speed

- LED status lights built into display

- TEAC floppy drive

- Gasket-sealed enclosures

- Jogstick for convenient jog

- 128 Mb USB Thumb Drive flash memory (O)

- Clean front panel with few hard keys

### Software Features – general operation

- Clear, uncluttered screen display

- Prompted data inputs

- English language – no codes

- Soft keys - change within context

- Windows® operating system

- Color graphics with adjustable views

- Inch/mm selectable

- Convenient modes of operation (see below)

### DRO Mode features

- Incremental and absolute dimensions

- Jog at rapid with override

- Powerfeed X or Z

- Tapers of any angle

- Radius

- Fillet

- Go To dimensions (O)

- Servo motor return to Home

- Spindle speed setting with override

- Tool offsets from library

- Fine/course handwheel resolution (O)

### Program Mode features

- Geometry-based programming

- Event comments (O)

- Incremental and absolute dimensions

- Automatic tool nose radius compensation

- Circular interpolation

- Linear interpolation

- Look –graphics with a single button push

- List step – graphics with programmed events displayed

- Alphanumeric program names

- Conrad – one input for automatic corner radius programming

- Chamfer – one input for automatic chamfer programming

- Math helps with graphical interface

- Auto load of math solutions

- Subroutine repeat of programmed events

- Nesting

- Programmable spindle speeds

- Program run time clock in Run Mode (O)

- CSS (Constant Surface Speed) programming (O)

- IPR (Inch per Revolution) programming (O)

- Gang tooling (O)

### Canned cycles

- Position

- Drill

- Bore

- Turn

- Arc

- Cycle

- Thread

- Groove

- Custom thread (O)

- Tap (O)

### Edit mode Features

- Delete events

- Erase program

- Spreadsheet editing (O)

- Global data change (O)

- Clipboard to copy events between programs (O)

### Set Up Mode Features

- Program diagnostics

- Advanced tool library

- Tool library file save

- Icon-prompted tool setting

- Tool offsets with modifiers

- Single tool set-up

- Gang tool set-up (O)

- Indexer tool set-up (O)

- Advanced diagnostic routines

- Software travel limits

- Tool path graphics with adjustable views

- Program run time estimation clock

### Run Mode Features

- CAM file program run

- Real time run graphics with tool icon

- Countdown clock to next pause or tool change (O)

- TRAKing of programs during program run (O)

- Automatic indexing for tool change (O)

- Gang tool operation (O)

### Program In/Out Mode Features

- Simple program storage to floppy

- CAM program converter

- Converter for prior-generation ProtoTRAK programs

- DXF/DWG file converter (O)

- Selection of file storage locations (O)

- Automatic file back-up routine (O)

- Preview graphics for unopened files (O)

- Networking (O)

Control Options

### Advanced Features Option

- Spreadsheet editing

- Global data change

- Clipboard to copy events to another program

- Event comments

- Program run time estimator

- CSS (Constant Surface Speed) programming

- IPR (Inch per Revolution) programming

- Gang tooling

- Additional Canned Cycles:

- Custom thread

- Tap

### Networking/Memory Option

- Directory/File/Folder Program organization

- Automatic file back up routine

- Preview Graphics for unopened files

- USB Thumb Drive flash memory, 128 MB or more

- Networking via RJ 45 port

### The DXF File Converter Option

Import and convert CAD data into ProtoTRAK programs

DXF or DWG files

Chaining

Automatic Gap Closing

Layer control

Drawing Line Edit

Easy, prompted process you can do right at the machine

### CAM Out Converter Option

Save ProtoTRAK files as CAM files for running on different controls

### TRAKing/Electronic Handwheels Option

TRAKing of programs during program run

Go To Dimensions

Selectable Fine/Coarse handwheel resolution